E-Bike Data Logger Documentation

For Thalchemy and the Bureau of Land Management

Prepared by Zander Alba and Chocie Landvik

April 2025

Section 1: Deployment and Maintenance

  - Deployment

  - UI installation and usage

1.3. Data Collection

1.4. Changing Batteries

1.5. Troubleshooting

Section 2: Hardware

2.1. Materials and sources

2.2. Schematics

Section 3: Software

3.1. Desktop Application

3.2. Microcontroller code

3.3. Machine Learning & Preprocessing

Section 1: Deployment and Maintenance 				    1.1 Deployment

In order to deploy a sensor array and be able to collect the most high-fidelity data, the following instructions must be followed.

- Ensure the battery is fully charged

- Burial location must be flat with no significant hills immediately before or after it from either direction of travel. This must be a non-motorized path that does not allow cars, ATVS or other motorized vehicles.

- A through 1.5 inches deep, 2 inches wide and 28 inches across must be dug perpendicular to direction of travel on a trail

- The sensor array must be placed in the hole with dirt packed around it, leaving about a third of the pipe exposed.

- The battery box must be just off of the trail, obscured from view by a bush, rocks or other natural items that blend in with the landscape

- Ensure that the battery is plugged in and that the data connection cable is visible within the box.

1.2 UI Installation and Usage

The User Interface (UI) is the application that will be used to access and collect data collected by the microcontroller. This application is only runnable on windows machines, but should not require any other dependencies or installations.

INSTALLATION:

- Navigate to zanderalbaz/EBike-Data-Logger

- Download “E-Bike_Data_Logger.zip”

- Extract the file

- Create a shortcut for the application and store it on your desktop

  - Right click

  - Show More Options

  - Create shortcut

  - Drag and drop the shortcut to your desktop

USAGE:

- For Collection: The data cable must be connected to your machine

- For Data Viewing: The data table does not have to be connected to view historically collected data

1.3 Data Collection

Collection of bike signatures will happen automatically, but in order to have access to this data, the following steps must be taken. If deployed correctly, this process should be straightforward and involve NO digging. It is NOT recommended to do this in wet conditions.

- Open the box containing the battery, data cable, and microcontroller

- Disconnect the data cable from the battery

- Plug this cable into your laptop (should already have the app installed see section 1.2 for reference)

- Open the E-Bike Data Logger Application on your machine

- Click “Collect Data”

- Type in the location of the sensor array (whatever name/descriptor you have for it)

- Press the “EN” button on the microcontroller

- Immediately after, click “Confirm Data Transfer” in the application

- Wait for the transfer confirmation window to appear (this will take a few seconds, your data is downloading

- Disconnect from your laptop and plug the data cable back into the battery

- Shut the battery box and make sure it is hidden.

1.4 Changing Batteries

The battery within a sensor array should last for a month without intervention. To maintain functionality, the battery must be replaced every thirty days. There are two batteries that are compatible with the sensor array.

- Ensure that the replacement battery is fully charged

- Open the battery box

- Unplug the data cable from the current battery and carefully remove the battery without disrupting any pins

- Plug the data cable into the new battery

- Put the battery back into the box under the microcontroller

- Close the box and make sure it is hidden

1.5 Troubleshooting

If pins become unplugged:

If connection not successful:

If transfer not successful:

Section 2: Hardware				    	        2.1 Materials and Sources

Materials used to create the sensor platform and where they were purchased from

| Function | Material | Source |
| --- | --- | --- |
| Microcontroller | ESP32 |  |
| Sensors | Magnetometer/Accelerometer | SparkFun |
| Protection/Body | 2” PVC pipe, 27in long |  |
| Wires | 12 gauge insulated wire, copper core |  |
| Storage | 4GB SD Card | Adafruit |

2.2 Schematics

Section 3: Software

All of the software is accessible through the github: zanderalbaz/EBike-Data-Logger

3.1 Desktop Application

The desktop application was created in Java using the Intellij IDE on a Windows machine with Oracle OpenJDK 23.0.1 with a language level 17. It requires jSerialComm-2.11.0.jar. The source code was compiled into an executable using jPackage.

PACKAGES & DEPENDENCIES, Libraries??

3.2 Microcontroller Code

The code for the ESP32 was coded in C++ and created in the Arduino IDE version 1.8.12. Libraries: Adafruit BusIO, Adafruit MLX90393, SparkFun 9DoF IMU Breakout – ICM 20948 – Arduino Library, Adafruit Unified Sensor in addition to the Built-in libraries.

3.3 Machine Learning & Preprocessing

The machine learning and preprocessing algorithms were scripted and trained in python using Jupyter Notebook, where they were ported to a .json file that we then included as a header in the microcontroller code. The preprocessing algorithm takes in a 18x50 table (18 columns over 50 collections) and returns summary satistics. The summary satistics used are average, standard deviation and upper and lower quartile. This reduces it to a 18x4 table, which is then flattened into a single row (1x72) to pass to the algorithm. The algorithm used is a XGBoost algorithm with the following parameters: XGBOOST PARAMETERS.